Key to 1hr Fire Rated Loft Access Pull Catch

- Access panel door tray manufactured from powder coated 0.9mm Zintec steel, cut and folded. The door tray was 75mm deep overall.

- Foam strip polyurethane open cell foam 6mm x 8mm

- Access panel frame manufactured from 1.2mm Zintec sheet steel. The 25mm picture frame was mitred at each corner.

- Access panel frame screw fixed to bracing within ceiling

- Gap filled with RW3 Rockwool and sealed with Selocrete Fire proof mastic.

- Mild steel continuous hinge welded to the door tray and bolted to the frame using M6 bolts, nuts and washers.

- Flush pull latch manufactured from steel.

- Full backing plate pop riveted to the door tray to conceal insulation manufactured from 0.9mm thick Zintec sheet steel

- Door tray filled with Rockwool 50mm thick RW3 insulation